| 1.0 Scope |  |  |  | 2.0 Resource Required |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 Purpose: Calculating Glass pull tons by sample weight 1.2 Area of Application: All annealed lines 1.3 Responsibility: Shift Packing Officer 1.4 Accountability: Section head & Production head |  |  |  | 2.1 Man: Shift Packing Officer 2.2 Machine: Weight scale, glass cutter & mesuring tape. 2.3 Material: Glass sample. |  |  |  |
| 3.0 Terms and Definition |  |  |  | 4.0 Key Performance Requirements |  |  |  |
| 3.1 TPD: Tons per day |  |  |  | 4.1 Quality: Precisely calculation of glass pull based on the glass drawn from the rolling machine. - EHS: No cut injury |  |  |  |
| 5.0 Frequency |  |  |  | 6.0 Competency Enhancement |  |  |  |
| 5.1 Once in shift & whenever requirement. |  |  |  | 6.1 Training on Pull calculation by glass weight activity to be given to all shift officers. 6.2 Visual posters are to be used for communication. |  |  |  |
| 7.0 Procedure |  |  |  |  |  |  |  |
| 7.1 Sequence of Operation |  |  |  |  |  |  |  |
| 7.1.1 Ask cutting team to call the glass sample from line. *Ensure the stability of line before calling the glass Sample. The sample will be given by cutting operator from the Grenzebach screen. |  |  |  |  |  |  |  |
| 7.1.2 Request the Glass sample from Grenzebach HMI with trim Intact (Shown in image). |  |  |  |  | Grenzebach screen Bentler M2 |  |  |
| 7.1.3 Verify the dimensions of the sample (length, width) (as shown in image). |  |  |  |  |  |  |  |
| 7.1.4 Move the glass to cutting table. (as shown in image) |  |  |  |  |  |  |  |
| 7.1.5 Cut the sample into 6 equal parts + 2 trims (left & right) (as shown in image) |  |  |  |  |  |  |  |
| 7.1.6 Press the “Zero” or “Tare” button to calibrate the weighing scale and set the baseline reading to zero before performing weight measurement. |  |  |  |  |  |  |  |
| 7.1.7 Put all the pieces of the glass on the weighing scale platform. (as shown in Figure-05) |  |  |  |  |  |  |  |
| 7.1.8 Note the weight of the sample which is shown on the display. |  |  |  |  |  |  |  |
| 7.1.9 Calculate the Glass pull by following the formula below: Glass pull (in tons/day) = (W * V * 1.44) / L Where: W: Weight of the sample in Kg. V: Lehr speed in m/min. L: Length of the sample in meter. |  |  |  |  |  |  |  |
| 7.2 Special Process Requirements |  |  |  |  |  |  |  |
| 7.3 Prevention & Detection Controls |  |  |  |  |  |  |  |
| 7.3.1 Identify any significant variation in the pull if needed and share the information with Hotend Shift in charge. |  |  |  |  |  |  |  |
| 7.4 Controls Related to EHS |  |  |  |  |  |  |  |
| 7.4.1 PPE MATRIX |  |  |  |  |  |  |  |
| Goggles | Safety shoes |  | Hand Gloves |  |  |  | Arm Guard |
| 7.4.2 After completion of activity, dispose of the glass sample in the cullet bin. |  |  |  |  |  |  |  |
| 8.0 EHS Compliance Obligations |  |  |  |  |  |  |  |
| 8.1 Not applicable |  |  |  |  |  |  |  |
| 9.0 Possible deviation & Impacts |  |  |  |  |  | 10. Proposed actions |  |
| 9.1 Improper sample size/weigh may lead the incorrect calculation 9.2 Improper use of formula may lead to wrong calculation & feedback |  |  |  |  |  | 10.1 Always “zero” set to weight machine prior to weight measurement & measurement of the sample size is necessary 10.2 Formula should be exact as mentioned in step 7.1.9 |  |
| 11. Reference documented information |  |  |  |  |  |  |  |
| 11.1 Daily pull data Annexure-1 |  |  |  |  |  |  |  |
| 12. Revision History |  |  |  |  |  |  |  |
| Revision No. | Date | Description of changes |  |  |  |  |  |
| 00 | 12.02.2020 | Initial release. |  |  |  |  |  |
| 01 | 01.07.2023 | Picture & safety requirement added |  |  |  |  |  |
| 02 | 08.07.2025 | Format standardized, revision history added, step correction done. |  |  |  |  |  |