| 1.0 Scope |  |  |  |  | - Resource Required |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Purpose: Internal cullet collection & handling - Area of Application: SG 1, 2, 3.1 & 3.2 1.3 Accountability: Cutting section in charge Annealing packing in charge Grinding section in charge Tempering section in charge Packing section in charge XY-Table in charge |  |  |  |  | 2.1 Man: Shift lehr & cutting officer Shift annealing packing line officer Shift grinding officer Shift tempering officer Shift packing officer & helpers 2.2 Machine: Forklift and Stacker 2.3 Material: Internal cullet, Racks, Jumbo bags, Wooden pallet and Metalic bins |  |  |  |  |
| 3.0 Terms and Definition |  |  |  |  | 4.0 Key Performance Requirements |  |  |  |  |
| 3.1 This standard operating procedure (SOP) outlines the proper procedures for handling internal cullet from various sources in a glass manufacturing facility to ensure safety, cleanliness, and efficient recycling or disposal of glass cullet. This SOP applies to all personnel involved in the handling of internal cullet at the glass manufacturing facility. |  |  |  |  | 4.1 Quality: To ensure the proper handling and management of internal cullet from various sources, maintaining safety standards and ensuring only clean cullet enters the crusher or batch-house. Consuming contaminated cullet will increase the defect level and decrease quality of glass. 4.2 EHS: Cut injuries. |  |  |  |  |
| 5.0 Frequency |  |  |  |  | 6.0 Competency Enhancement |  |  |  |  |
|  |  |  |  |  | 6.1 Training is to be given to all personnel involved in cullet handling must undergo training on this SOP. 6.2 Visual posters to be used for communication. |  |  |  |  |
| 7.0 Procedure |  |  |  |  |  |  |  |  |  |
| 7.1 Sequence of Operation |  |  |  |  |  |  |  |  |  |
| - Source of internal cullet: The sources identified here are for internal cullet generation. 7.1.1.1 from cold end: a. Rejected glass on rack: Ensure that rejected glass is placed securely on designated racks with weight not more than 1600kg. b. Cullet below conveyor line: Remove the cullet from below the conveyor line and transfer it to designated storage containers. If the cullet is dusty transfer it to Batch house for washing. c. Online breakage coming from lehr: Collect any glass breakage occurring during the online process and transfer it to designated storage containers. 7.1.1.2 From grinding line: a. Rejected sheet by QA: Rejected glass sheets from the grinding line shall be placed in designated racks with weight not more than 1600kg. b. Breakage of glass by robot and bentler: Glass breakage caused by robots or bentler’s shall be collected and placed in designated containers. c. Cullet below conveyor line: Any cullet found below the conveyor line in the grinding area shall be collected and placed in designated containers and if there is dusty cullet transfer it to Batch house for washing. 7.1.1.3 After ARC before tempering: a. Rejected sheet by QA: Rejected glass sheets after the arc but before tempering shall be placed in designated racks with weight not more than 1600kg. b. Cullet from dumping box: Glass cullet from the dumping box shall be collected and placed in designated containers. c. Cullet below conveyor line: Any cullet found below the conveyor line in this area shall be collected and placed in designated containers and transfer it to batch house for washing. 7.1.1.4 After washing and end packing: a. Rejected sheet by QA: Rejected glass sheets after washing and end packing shall be placed in designated racks. b. Cullet below conveyor line: Any cullet found below the conveyor line in this area shall be collected and placed in designated containers if there is dusty cullet transfer it to batch house for washing.. 7.1.1.5 Cullet from Lehr (Dusty Cullet) & XY-Table: a. Clean cullet from XY-Table: Clean cullet from the XY-table shall be sent directly to the crusher. b. Cullet from lehr put into bags: Cullet from the Lehr shall be placed into bags and sent to the batch-house for washing. c. Dusty Cullet in bags: Dusty cullet shall be placed in bags and sent to the batch house for washing. |  |  |  |  |  |  |  |  |  |
| 7.1.2 Precautions for dumping: a. Proper PPEs (Hand gloves, arm guard, goggles, mask, safety shoes) shall be worn at all times. b. Nearby areas shall be kept free from cullet and glass dust. c. Ensure that the cullet being dumped does not contain PPEs, plastic, wood, paper, or metallic parts. d. Pallet/rack should not be in a lifting position during dumping. e. Only clean cullet should be dumped into the crusher, and dusty cullet should be sent to the batch house in bags for cleaning. f. Stacking OK or rejection sheets should not exceed the load capacity of 1600 kgs. g. In case of an emergency with the crusher, push the emergency switch. |  |  |  |  |  |  |  |  |  |
| 7.1.3 Mode of transit of cullet: a. Rejected sheets shall be placed on racks by the stacker. b. Cullet shall be transported by manual metallic trolley. c. Rejected sheets from end packing placed on pallets by the stacker. d. Ensure that pallet/rack is not overloaded i.e.1600kg (on 3.2mm base M10 size - 80 pcs/rack, 1900 series - 100 pcs/rack, 1700 series - 110 pcs/rack). |  |  |  |  |  |  |  |  |  |
| 7.1.4 Method of dumping into cullet crusher: a. When dumping rejected sheets on racks/pallets, dump one sheet at a time. b. Slowly push the cullet by unlocking the pin of the manual metallic trolley. c. Ensure that only clean cullet is dumped into the crusher. |  |  |  |  |  |  |  |  |  |
| 7.1.5 Records and documentation: - Record the glass cullet data for each location. - Analyze the internal cullet data to identify any significant variations or deviations from the acceptable range. - Share the report with relevant stakeholders, such as production personnel or process engineers. - Maintain records of cullet handling incidents, including any accidents, near misses, or deviations from this SOP. |  |  |  |  |  |  |  |  |  |
| 7.1.6 Review: This SOP will be reviewed annually or as needed to ensure its effectiveness and relevance. |  |  |  |  |  |  |  |  |  |
| 7.2 Special Process Requirements |  |  |  |  |  |  |  |  |  |
| 7.2.1 Insert Procedure text in first person |  |  |  |  |  | Insert respective pictures here |  |  |  |
| 7.2.2 |  |  |  |  |  |  |  |  |  |
| 7.3 Prevention & Detection Controls |  |  |  |  |  |  |  |  |  |
| 7.3.1 (Add list of checks, monitoring and measurement points) 7.3.2 |  |  |  |  |  |  |  |  |  |
| 7.4 Controls Related to EHS |  |  |  |  |  |  |  |  |  |
| 7.4.1 PPE MATRIX |  |  |  |  |  |  |  |  |  |
| Arm guard |  | Goggles |  | Nose mask |  |  |  | Safety shoes | Hand gloves |
| 7.4.2 Add EHS controls as applicable under separate headings: E.g. Waste disposal, Personnel exposure, Ergonomics, chemical safety, spill prevention, fire prevention, injury prevention etc.. |  |  |  |  |  |  |  |  |  |
| 7.4.3 |  |  |  |  |  |  |  |  |  |
| 8.0 EHS Compliance Obligations |  |  |  |  |  |  |  |  |  |
| 8.1 List all legal & other requirements applicable with titles of rules / consents etc (consult EHS/utilities / IMS teams for more clarity) |  |  |  |  |  |  |  |  |  |
| 9.0 Possible deviation & Impacts |  |  |  |  |  |  | 10. Proposed actions |  |  |
| 9.1 If prompt action is not taken in cases of internal cullet handling, the glass's quality and efficiency will be impacted |  |  |  |  |  |  | 10.1 Check the internal cullet quality, generation, size, etc., and take the appropriate action. |  |  |
| 11. Reference documented information |  |  |  |  |  |  |  |  |  |
| 11.1 Refer all cross-reference documentation & records and formats to be used for this SOP |  |  |  |  |  |  |  |  |  |
| 12. Revision History |  |  |  |  |  |  |  |  |  |
| Revision No. | Date |  | Description of changes |  |  |  |  |  |  |